Annex 9 – Final Documentation

1	General

All documentation shall be provided in electronic format.

For PDF documents where A4 size will be not enough to appreciate all the details and read all the letters, such as layouts and maps, bigger document sizes must be chosen.

Documentation shall be provided in accordance with the requirements of IEC62446.

2	O&M Manual

The Site Manual should be held on site and kept up to date throughout the project.

Owner name & address (SPV)

Project name

Project Start date.

Site address including post code

GPS coordinates (of site entrance and Switchgear building)

Rated system power (kW DC) (Total Installed Capacity)

Total AC Capacity (Declared Net Capacity) (kVA AC) and Connection Voltage

Hazardous Voltages Present (DC & AC)

System Designer, (company, contact person, phone & address)

Emergency Phone Numbers

O&M Operator

Senior Authorized Person in charge of switching operations

Health and Safety Officer contact details

Distribution Network Operator (DNO)

Security Company

Energization Target Date.

Manufacturer, model, serial number rated power and quantity of:

Solar Modules

Inverters

Transformers

Meter Model, Standards Accuracy & Serial Nos (Main and Backup)

List all contractors (signed H&S information board should be displayed in the site office)

General Layout

Single Line Diagram

List of latest drawings (including version number)

3	Documentation Required for Technical Design (Executive Design)

The following set of documentation should be made available by the Contractor inside the Executive Design to be issued:

1.1.	Structural calculations for all structures and buildings (according NCh 2369 or NCh433 or UBC/IBC) including (as applicable to the installation):

a.	Soil Mechanics taking into account:

i	Clear identification of the stratum suitable for foundation or the characteristics of the filling soil suitable for foundation (Parameters for calculation of admissible stresses and stability for any foundation geometry)

ii	Specific weight ϒs

iii.	Cohesion ς

iv.	Internal Friction Angle Φ

v.	Poisson module ν

b.	Equation to determine admissible static and tension (for any geometry), from where q adm‐static can be obtained

c.	Minimum level of seal of foundation.

d.	Type and characteristics of structural filling, if necessary, and for what type of elements (roads, foundations, etc.)

e.	Frost considerations and expected frost level depth.

f.	Slopes and recommended cuts.

g.	Civil‐structural design criteria:

i.	Standards and codes (National, foreign and others)

ii.	Design loads (permanent, overload or use, wind, seismic, equipment, assembly, etc.)

iii.	Combinations of loads (ASD, LRFD)

iv.	Admissible deformations.

v.	Materials (characteristics, qualities, etc.)

vi.	Criteria for the design of foundations (safety factors for tipping, sliding, minimum compressed area)

vii.	Design criteria for reinforced concrete.

viii.	Design criteria for metal structures

ix.	Drainage design criteria (if applicable). Road design criteria.

x.	Enclosures and gates.

h.	Technical Specifications of Reinforced Concrete:

i.	Standards and definitions (cement, aggregate, armors, etc.)

ii.	Materials (cement, water, aggregates, storing, additives, sealants, waterproofing)

i.	Technical Specifications for steel structures

i.	Standards and definitions (cement, aggregate, armors, etc.)

ii.	Materials

iii.	Painting scheme

j.	Specifications for paints and coatings:

i.	Standards and definitions

ii.	Description of the products

iii.	Application conditions

iv.	Protection schemes

v.	Paint schemes

k.	Standard plan for reinforced concrete, must contain at least, and as appropriate: (if necessary, it can be more than one sheet)

i.	General notes

ii.	Materials

iii.	Coatings

iv.	Typical details

v.	Abbreviations

vi.	Symbologies

l.	Standard layout for metallic structures must contain:

i.	General notes

ii.	Materials

iii.	Typical details

iv.	Abbreviations

v.	Symbologies

1.2.	ELECTRIC ENGINEERING

a.	Equipment list

b.	Cable List.

c.	Technical Specification for main Equipment. (for quotes)

d.	Datasheet (all Electrical Equipment).

e.	Drawings of underground electrical

f.	Signed and dated low voltage DC circuit functional tests in accordance with IEC 62446 (BS EN 62446:2009) 2016.

g.	Sizing calculation reports: Lines, cables, ground mesh, transformers, etc.

i.	Protection coordination study (selectivity).

ii.	MV Single Line diagrams (General and protections) (according NCh 4, NSEG5_71 & NTCO)

iii.	LV Single Line diagrams (General of Auxiliary Services) (according NCh 4, NSEG5_71 & NTCO)

h.	Interconnection plans Inverters‐Transformers‐ Switchgear.

i.	Interconnection plans for the modules‐Box combiner‐inverters.

j.	Geoelectric studies.

k.	Plans and memories of calculation of earth meshes.

l.	Earthing design including layout and calculations (according IEEE 80‐2013)

m.	Design by IEC62548 Array Design (or 62738 Power Plant) In accordance to Scope of works.

1.3.	Cable sections

a.	Cable section calculations (according to local codes and standards and also some international ones such as IEC 60364)) including:

i.	Short‐circuit capacity

ii.	Load current capacity

iii.	Voltage drop

b.	Short‐circuit calculations (according IEC 60909) including:

i.	Short‐circuit level for all bus bars

c.	Electric protections study (according IEEE Std 242‐2001, Std 399‐1997 & Std C57.109‐2008 & IEC 60076‐5‐ 2006) including:

i.	Adjustment and coordination of protection systems of the plant

1.4.	Civil works layouts and designs:

a.	Trenches Layout (according NCh 4)

b.	Detailed equipment layout plans.

4	Documentation Required for Technical Acceptance

The following set of documentation should be made available by the Contractor at the time of issuing the Mechanical Completion Certificate and request for Technical Acceptance.

4.1	Up to date versions of SLD, General Site Layout & Cable Trench Layout including module mapping layout.

4.2	Summary of changes between "As built" and the latest revision

4.3	List of any remaining works that still needs to be completed but, in the view of the Contractor, do not impact the commissioning of the Plant.

4.4	Static/dynamic loading calculations for the Support Structure performed by the manufacturer.

4.5	Galvanising certificates for the support structures and results of pull out tests.

4.6	Product documentation (Datasheets, manuals and copies of shall be provided for the major items (Solar modules, Support Structure Inverter, Transformer's, Switchgear)

4.7	Warranty certificates of the major items (modules, inverters, structure, transformer & switchgear) with reference to the serial numbers of the products installed (where applicable)

4.8	Declaration of Total Installed capacity (TIC) and Declared Net capacity (DNC)

4.9	Photovoltaic modules flash test results.

4.10	Grid Connection Offer letter send by the DNO and the acceptance letter sent by the developer in response. Both documents reserve the grid capacityfor the current project.

5	Documentation Required for Commissioning

In addition to the documentation required for Section 3, the following set of documentation should be made available by the Contractor when claiming that the Commissioning of the site is completed.

5.1	Signed and dated commissioning document issued by the EPC contractor related to all MV and LV equipment installed at the plant. Any related commissioning procedure, testing protocols and testing results should be included.

5.2	Latest versions of SLD, General Site Layout & Cable Trench Layout including module mapping layout..

5.3	Signed and dated low voltage DC circuit functional tests in accordance with IEC 62446 (BS EN 62446:2009)

5.4	Inverter commissioning protocols (for both cold and hot commissioning) including any limitation implemented by software.

5.5	Signed and dated switchgear insulation test results and commissioning protocols.

5.6	Signed and dated Insulation Resistance test report (i.e. IR/cable pressure test) of Migh Voltage circuits.

5.7	The insulation of electric circuits and continuity according to IEC 60364 performed and signed and dated by a qualified and competent contractor.

5.8	Calibration certificates for all instrumentation used during the performance of all the tests.

5.9	Location and details of all electrical metering including meter type (make and model) and serial numbers

5.10	Signed and dated cold commissioning report of Plant Disconnection Relay from certified engineer verifying the parameters required by the DNO, the "Formulario 9".

5.11	Energization certificate, the "Formulario 10".

5.12	Signed and dated version of the following contracts: Grid Connection Agreement, Adoption Agreement, Completion Certificate under Contestable Works and Joint Operating Agreement (if applicable).

5.13	Copies of all forms and studies presented to the DNO.

5.14	Detailed design calculations and drawings for earthing circuits.

5.15	Earthing test reports of Client substation and the PV side.

5.16	List of any remaining works that still need completing prior to starting the Provisional Acceptance Test and/or issuing of Provisional Acceptance Certificate complete with timeline for completion.

6	Documentation required for issuance of Provisional Acceptance Certificate

In addition to the documentation required for Section 4, the following set of documentation should be made available by the Contractor when claiming that the Provisional Acceptance is completed.

6.1	Complete set of As Built Drawings mutually agreed between the 2 parties.

6.2	Commissioning report signed by the installer/manufacturer & complete set of documentation for the monitoring system

6.3	Commissioning report signed by the installer/manufacturer & complete set of documentation for the Security system

6.4	The calibration certificates for the Pyranometers

6.5	Operation and maintenance information including,

6.5.1	Procedures for verifying correct system operation,

6.5.2	A checklist of what to do in case of a system failure.

6.5.3	Emergency shutdown / isolation procedures.

6.5.4	Maintenance and cleaning recommendations (if any).

6.6	Spare parts list with the amount of each item.

6.7	Health and Safety file defined as the site health and safety records pertaining to and made during construction and confirmation of site incident statistics.

6.8	Quality inspection reports

7	Requirements for General Layout

General layout including the geospatial coordinates taken by a certified topographer, and showing the exact position of all the elements of the plant including panels, structures, string boxes, inverters, transformers, cabins, MV cables, LV cables, earth cables, earth spears, fence, cameras, security systems, monitoring systems, security sensors, access doors, roads, manholes, cable ducts, conduits, draining elements, connection cables, delivery cabin, substation, meter, near roads, trees, hedges, land plots, and others.

| Item | Area | Cat | Document Code | Description (Rev 02) | Revision | Status | Type | Elaborado/Prepar ado por: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PV | ELE | as per QA/QC Plan | LV DC/AC SLD | 00 | IFC | DRW | CONTRACT |
| 2 | PV | ELE | as per QA/QC Plan | Overall SLD (DC_LV-AC_MV_OH) | 00 | IFC | DRW | CONTRACT |
| 3 | PV | CTRL | as per QA/QC Plan | SCADA architecture drawings | 00 | IFC | DRW | CONTRACT |
| 4 | PV | ELE | as per QA/QC Plan | Strings cables and Inverters installations details | 00 | IFC | DRW | CONTRACT |
| 5 | PV | ELE | as per QA/QC Plan | LV AC/DC cable layout & details | 00 | IFC | DRW | CONTRACT |
| 6 | PV | CIV | as per QA/QC Plan | Layout MV & LV Trenches cross Sections & details | 00 | IFC | DRW | CONTRACT |
| 7 | PV | CIV | as per QA/QC Plan | Fence and gate layout and sections | 00 | IFC | DRW | CONTRACT |
| 8 | PV | CIV | as per QA/QC Plan | Drainage system mechanical calculation (if applicable) | 00 | IFC | REP | CONTRACT |
| 9 | PV | CIV | as per QA/QC Plan | Layout Roads & details | 00 | IFC | DRW | CONTRACT |
| 10 | PV | CIV | as per QA/QC Plan | Hydrology report update (If applicable) | 00 | IFC | REP | CONTRACT |
| 11 | PV | CIV | as per QA/QC Plan | Temporary facilities layout and SLD | 00 | IFC | DRW | CONTRACT |
| 12 | PV | CIV | as per QA/QC Plan | Geotechnical Study (If Applicable) Mecanica de Suelos | 00 | IFC | REP | CONTRACT |
| 13 | PV | ELE | as per QA/QC Plan | Protection coordinations study / including MV relay settings | 00 | IFC | REP | CONTRACT |
| 14 | PV | ELE | as per QA/QC Plan | Grounding study | 00 | IFC | REP | CONTRACT |
| 15 | PV | ELE | as per QA/QC Plan | Grounding Layout & details | 00 | IFC | DRW | CONTRACT |
| 16 | PV | GEN | as per QA/QC Plan | O&M Building Layout | 00 | IFC | DRW | CONTRACT |
| 17 | PV | CIV | as per QA/QC Plan | MV-Station, O&M & Waste Tanks foundation drawings | 00 | IFC | DRW | CONTRACT |
| 18 | PV | CIV | as per QA/QC Plan | MV-Station, O&M, Waste Tanks, Inverters and CCTV foundation calculations report | 00 | IFC | REP | CONTRACT |
| 19 | PV | CIV | as per QA/QC Plan | Drainage system layout & details (if applicable) | 00 | IFC | DRW | CONTRACT |
| 20 | PV | CCTV | as per QA/QC Plan | CCTV Layout & Poles details | 00 | IFC | DRW | CONTRACT |
| 21 | DCON | GEN | as per QA/QC Plan | Master Documents List | 00 | IFC | DOC | CONTRACT |
| 22 | PV | MEC | as per QA/QC Plan | Tracker Manufacturer Documents Package | 00 | IFC | DOC | CONTRACT |
| 23 | QA | GEN | as per QA/QC Plan | Quality Plan | 00 | IFC | DOC | CONTRACT |
| 24 | HS | GEN | as per QA/QC Plan | Environmental, Health & Safety plan | 00 | IFC | DOC | CONTRACT |
| 25 | PV | GEN | as per QA/QC Plan | General lay-out | 00 | IFC | DRW | CONTRACT |
| 26 | PV | GEN | as per QA/QC Plan | Overall KMZ location | 00 | IFC | DRW | CONTRACT |
| 27 | PV | ELE | as per QA/QC Plan | Cable Sizing and Energy Losses Calculation | 00 | IFC | REP | CONTRACT |
| 28 | PV | ELE | as per QA/QC Plan | PVSyst simulation | 00 | IFC | REP | CONTRACT |
| 29 | PV | CIV | as per QA/QC Plan | Grading (levelling) plans | 00 | IFC | DRW | CONTRACT |
| 30 | PV | CIV | as per QA/QC Plan | O&M Building Manufacturer Documents Package | 00 | IFC | DRW | CONTRACT |
| 31 | QA | CIV | as per QA/QC Plan | Compacting test protocol from lab | 00 | IFC | DOC | CONTRACT |
| 32 | PV | GEN | as per QA/QC Plan | Listings of materials | 00 | IFC | DOC | CONTRACT |
| 33 | PV | GEN | as per QA/QC Plan | Cable listings / Cable Routing | 00 | IFC | DOC | CONTRACT |

| Item | Area | Cat | Document Code | Description (Rev 02) | Revision | Status | Type | Elaborado/Prepar ado por: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 | PV | GEN | as per QA/QC Plan | MV Station Manufacturer Documents Package | 00 | IFC | DRW | CONTRACT |
| 35 | PV | ELE | as per QA/QC Plan | Auxiliary circuits SLD | 00 | IFC | DRW | CONTRACT |
| 36 | PV | CIV | as per QA/QC Plan | Fence and gate Manufacturer Documents Package | 00 | IFC | DOC | CONTRACT |
| 37 | PV | GEN | as per QA/QC Plan | Inverter Manufacturer Documents Package | 00 | IFC | DOC | CONTRACT |
| 38 | PV | GEN | as per QA/QC Plan | PV-Module Manufacturer Documents Package | 00 | IFC | DOC | CONTRACT |
| 39 | PV | ELE | as per QA/QC Plan | Short-Circuit Study | 00 | IFC | DOC | CONTRACT |
| 40 | PV | CTRL | as per QA/QC Plan | SCADA field panel wiring details | 00 | IFC | DRW | CONTRACT |
| 41 | PV | CTRL | as per QA/QC Plan | Monitoring System / Control Systems SLD | 00 | IFC | DRW | CONTRACT |
| 42 | PV | CTRL | as per QA/QC Plan | Monitoring System Manufacturer Documents Package | 00 | IFC | DOC | CONTRACT |
| 43 | PV | CTRL | as per QA/QC Plan | SCADA System configuration details (IPs, Modbus, ports, passwords, bus speeds and other parLYNKers) O&M Manual | 00 | IFC | DOC | CONTRACT |
| 44 | PV | GEN | as per QA/QC Plan | CCTV Foundation Calculation | 00 | IFC | REP | CONTRACT |
| 45 | PV | CCTV | as per QA/QC Plan | CCTV Equipments SLD | 00 | IFC | DOC | CONTRACT |
| 46 | PV | CTRL | as per QA/QC Plan | Monitoring System: List of Alarms (including type, delays, area, device and other details) | 00 | IFC | DOC | CONTRACT |
| 47 | PV | CTRL | as per QA/QC Plan | O&M Manual (with cover, general notes, abbreviations and symbols) | 00 | IFC | DOC | CONTRACT |
| 48 | PV | ELE | as per QA/QC Plan | Commissioning Test Procedures | 00 | IFC | DOC | CONTRACT |
| 49 | PV | GEN | as per QA/QC Plan | CCTV Specification | 00 | TBD | TBD | CONTRACT |
| 50 | PV | GEN | as per QA/QC Plan | CCTV Manufacturer Documents Package | 00 | TBD | TBD | CONTRACT |
| 51 | PV | GEN | as per QA/QC Plan | RFQ/Specification Cables (All Cables) | 00 | TBD | TBD | CONTRACT |
| 52 | PV | CTRL | as per QA/QC Plan | RFQ/Specification SCADA System | 00 | TBD | TBD | CONTRACT |
| 53 | PV | GEN | as per QA/QC Plan | RFQ/Specification O&M Building | 00 | TBD | TBD | CONTRACT |
| 54 | PV | GEN | as per QA/QC Plan | RFQ/Specification Wastes Tanks | 00 | TBD | TBD | CONTRACT |
| 55 | PV | GEN | as per QA/QC Plan | RFQ/Specification Fence | 00 | TBD | TBD | CONTRACT |
| 56 | PV | GEN | as per QA/QC Plan | Project execution Plan | 00 | IFC | DOC | CONTRACT |
| 57 | PV | GEN | as per QA/QC Plan | Cables Manufacturer Documents Package | 00 | TBD | TBD | CONTRACT |
| 58 | PV | GEN | as per QA/QC Plan | Spare parts List | 00 | IFC | DOC | CONTRACT |
| 59 | PV | GEN | as per QA/QC Plan | Interconnection Diagram Details | 00 | TBD | TBD | CONTRACT |
| 60 | QA | TBD | as per QA/QC Plan | Plant Acceptance Test Procedure (Including PR) | 00 | TBD | TBD | CONTRACT |
| 61 | PV | GEN | as per QA/QC Plan | Wastes Tanks Manufacturer Documents Package | 00 | TBD | TBD | CONTRACT |
| 62 | PV | CTRL | as per QA/QC Plan | SCADA System Manufacturer Documents Package | 00 | TBD | TBD | CONTRACT |
| 63 | PV | CIV | as per QA/QC Plan | Topographic Report Update (If Applicable) | 00 | IFC | REP | CONTRACT |
| 64 | PV | ELE | XXX-PV-ELE-DOC-CONTRACT-701-00 | Site Acceptance Test Plan | 00 | IFC | DOC | CONTRACT |